[image: image1.jpg]



CLAYWORKS WOOD FIRED OVENS


THE COMPLETE KIT

Purchase the complete kit and you will receive:

· The Basic Unit

· Refractory Floor

· Refractory Outer Skin

· Support Stand

· Detailed Installation and Firing Instructions

With the complete kit two capable people can have the oven installed and the fire lit in approximately an hour.  No need for a crane or forklift, the segments are suitable for two people to install.  The estimated time for installation of the oven assumes that a suitable site is available.

SITE PREPARATION

A flat area 1.2 by 1.2 metres is required for the oven.  We suggest that it is levelled with crushed rock, concrete, paver or similar surface treatment.  An additional, cleared flat area is required in front of the oven to allow ready and safe access during cooking.  Please check with your local fire authority.

To comply with local fire regulations it maybe necessary to clear all combustible materials from the area immediately adjacent to the oven and to have a water supply available.

Site preparation is not included in the scope of supply by Clayworks.

INSTALLATION OF THE OVEN

Installation is very simple and can easily be completed by two capable people.  Follow the Detailed Installation Instructions included with the kit; the following is an overview of the procedure:

The SUPPORT BASE:

Included with the kit are:

· The Stand

· Cement Sheet

· Insulation Board

· High Temperature Tiles

Each of the components is cut to size and ready for immediate installation.

To install the Support Base:

· Locate THE STAND on your prepared area in its final position, using a spirit level ensure that its is level. 

· Place the Cement Sheet on the supporting rails of the stand.  The sheet should fit snug and level, however, a 5mm gap along any edge is acceptable.

· Lay the Insulating Boards on top of the Cement Sheet

· Place the High Temperature Tiles on top of the Insulating Boards.

The BASIC UNIT:

Included with the kit are the cast segments of the Basic Unit, as shown:

[image: image9.png]FRear

PartF

PartD

Fromt





[image: image2.jpg]



[image: image3.jpg]



[image: image4.jpg]



        Part “A”
         Part “B”
         Part “C”
      Part “D”

Plus the following items:

· Ceramic gaskets

· High Temperature Silicone

· Double skin flue, with Cowl.

· Door

PLEASE NOTE, when handling the cast segments we strongly recommend that heavy duty gloves are worn.

[image: image7.png]Rear

Partc

PartD

Front





To Install the Basic Unit, starting by placing Part A on the SUPPORT BASE, followed by sections B and C.  Between each segment a ceramic gasket is inserted.  Ensure the oven is centrally located on the base.
DO NOT install Part D at this stage.

The REFRATORY OUTER SKIN

Included with the kit are the cast segments of the Refractory Outer Skin, as shown:

[image: image5.jpg]



[image: image6.jpg]



      Part “E”
 Part “F”

To install the outer skin:

· Place part E on the stand; slide it forward until none of it is overhanging the back edge.  

· Place part F on the stand; slide it back until it meets Part E.

[image: image8.jpg]



Completing Installation

To complete installation:

· Place Part D on the Support Stand and slide into position

· Place the cowl on the flue.

· Install the complete chimney into Part D, using a spirit level to ensure it is vertical.

· Seal the gap, where the chimney meets Part “D” using the High Temperature Silicone.

· Put the door in place.

The Oven is now ready for the fire to be lit; we recommend the unit be fired prior to using it to cook food.  Please refer to the firing instructions.







CLAYWORKS POTTERS SUPPLY PTY LTD

6 Johnston Court, Dandenong, Victoria, 3175

tel: 03 97916749  fax 03 97924476


